Measurement Notes & Test Conditions for 506WLS UBL SMT Inductors

This describes the applicable test conditions and equipment used in the measurement of scattering parameters for the ATC 506WLS Ultra Broadband Surface Mount Inductors.

The 506WLS UBL SMT inductor is composed of the pyramidal inductor coil and the mounting support structure, hereafter referred to as the Unit Under Test (UUT). The UUT is mounted in a shunt configuration on a microstrip. The UUT is oriented with the inductor tip point on the center length of the trace line, in such a way to have the UUT perpendicular to the trace length (Figure 1).

Figure 1

Substrate information is given in Table 1.

| Substrate | Substrate Length | Substrate Thickness | Trace Width | Dielectric Constant (εr) |
| --- | --- | --- | --- | --- |
|  | L (mils) | T (mils) | W (mils) |  |
| Rogers 4350B | 663 | 10 | 22 | 3.66 |

Table 1

At the tip end of the inductor, solder was applied 10 mils away from the coil winding going to the wire loop that is connected to the mounting base (Figure 2).

Figure 2

The base end of the inductor was soldered using a jumper wire through a via hole to the ground plane (Figure 3).

Figure 3

Measurements were collected using an Agilent N5230A Vector Network Analyzer (VNA). The VNA has a four-receiver architecture. All measurements used an Anritsu 3680K Universal Test Fixture. De-embedding was accomplished by using a standard TRL calibration.

Data was taken at 401 points between 400 MHz to 40 GHz. A snippet of the data format is shown in table 2. Table 2 does not reflect actual measurements made on the UUT.

| # MHz S DB R 50 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 400 | -10.812 | 71.292 | -0.4176 | -17.821 | -0.4227 | -17.915 | -10.767 | 71.097 |
| 423 | -10.384 | 70.343 | -0.4643 | -18.784 | -0.4605 | -18.844 | -10.334 | 70.248 |
| 445 | -9.9713 | 69.478 | -0.5057 | -19.74 | -0.5039 | -19.778 | -9.9209 | 69.355 |

Table 2

The first line is a header, designated by the “#” character. Succeeding characters describe respectively the frequency units, parameter, measurement format, and characteristic impedance of the measurement system (in this case 50 ohms).

The data is arranged as follows: The first column is the frequency in MHz. The next two columns are S11 magnitude (in dB) and S11 phase angle (in degrees).  Succeeding column pairs represent S21, S12, and S22.

DISCLAIMER: American Technical Ceramics (ATC) has made every effort to make this information as accurate as possible.  However, no responsibility is assumed by ATC for its use, nor any infringements of rights of third parties which may result from its use.  ATC reserves the right to revise the content or modify its product line without prior notice.