ATC 0402, 0603, 0805 WL Series Measurement Notes and Test Conditions

This describes the applicable test conditions and equipment used in the measurement of scattering parameters for the ATC 0402, 0603, and 0805 WL series inductors.

Each inductor was mounted in a series configuration on microstrip.  As shown below, each unit under test (UUT) subtended a “G”-mil gap in a “W”-mil wide trace on Rogers 4350 softboard, “T” mils thick.  “G,” “W,” and ”T” are given in Table 1 for various-size inductors.  In general, “W” was kept at about the width of the inductor, with “T” selected to make the trace width a 50-ohm line.

| Case Size | G (mils) | W(mils) | T(mils) |
| --- | --- | --- | --- |
| 0402 | 24 | 22 | 10 |
| 0603 | 36 | 29 | 13.3 |
| 0805 | 46 | 52 | 23.3 |

All measurements were made using an Anritsu 3680K Universal Test Fixture and an HP8722D Vector Network Analyzer having a four-receiver architecture.  Measurements have been de-embedded to the edges of the UUT using a standard TRL calibration procedure.  (I.e., S-parameters are referenced at the edges of each inductor.)

Data is taken at 401 points for each inductor value.  The following is a data sample:

#  HZ  S  DB  R  50

0.400  -10.812    71.292   -.4176   -17.821   -.4227   -17.915   -10.767    71.097

0.422  -10.384    70.343   -.4643   -18.784   -.4605   -18.844   -10.334    70.248

0.445  -9.9713    69.478   -.5057   -19.74     -.5039   -19.778   -9.9209    69.355

The first line is a header, designated by the “#” character. Succeeding characters describe respectively the frequency units, parameter, measurement format, and characteristic impedance of the measurement system (in this case 50 ohms).

The data is arranged as follows: The first column is the frequency in GHz.  The next two columns are S11 magnitude (in db) and S11 phase angle (in degrees).  Succeeding column pairs represent S21, S12, and S22.

The magnitude of each S-parameter for passive devices must be equal to or less than unity, and when expressed in dB, it must be    However, because of small, unavoidable errors in the TRL calibration, the magnitudes of some measured S-parameters near 0 dB are occasionally slightly positive.  In the published scattering matrices, each of these elements is assigned a value of  -0.01 dB.

Some RF design simulation software programs require that S-Parameter values be specified at zero frequency. The following zero frequency values have been added to satisfy this requirement.

S11, S22 Magnitude (dB) = -999, S11, S22 Phase angle = 90 degrees.

S12, S21 Magnitude (dB) = 0.  S12, S21 Phase angle = 0 degrees.

DISCLAIMER: American Technical Ceramics (ATC) has made every effort to make this information as accurate as possible.  However, no responsibility is assumed by ATC for its use, or any infringements of rights of third parties, which may result from its use.  ATC reserves the right to revise the content or modify its product line without prior notice.